hazard_detection_unit.sv

module hazard_detection_unit( input Memread, input [31:0] inst, input [4:0] Rd, output reg stall ); initial begin stall = 1'b0; end always @(*) begin if (Memread == 1'b1 && ((Rd == inst[19:15]) ¦¦ (Rd == inst[24:20]))) stall = 1'b1; else stall = 1'b0; end endmodule

reg_execute_memory.sv

module reg_execute_memory( input clk, input reset, input [63:0] Adder_out, //adder output input [63:0] Result_in_alu, //64bit alu output input Zero_in, //64bit alu output input [63:0] writedata_in, //2 bit mux2by1 output input [4:0] Rd_in, //IDEX output input branch_in, input memread_in, input memtoreg_in, input memwrite_in, input regwrite_in, //IDEXX outputs input flush, input addermuxselect_in, output reg [63:0] Adderout, output reg zero, output reg [63:0] result_out_alu, output reg [63:0] writedata_out, output reg [4:0]rd, output reg Branch, output reg Memread, output reg Memtoreg, output reg Memwrite, output reg Regwrite, output reg addermuxselect ); always @ (posedge clk) begin if (reset == 1'b1 ¦¦ flush == 1'b1) begin Adderout = 64'b0; zero = 1'b0; result_out_alu = 63'b0; writedata_out = 64'b0; rd = 5'b0; Branch = 1'b0; Memread = 1'b0; Memtoreg =1'b0; Memwrite = 1'b0; Regwrite = 1'b0; addermuxselect = 1'b0; end else begin Adderout = Adder_out; zero = Zero_in; result_out_alu = Result_in_alu; writedata_out = writedata_in; rd = Rd_in; Branch = branch_in; Memread = memread_in; Memtoreg = memtoreg_in; Memwrite = memwrite_in; Regwrite = regwrite_in; addermuxselect = addermuxselect_in; end end endmodule

control_unit.sv

module control_unit( input [6:0] opcode, input stall, output reg branch, output reg memread, output reg memtoreg, output reg memwrite, output reg aluSrc, output reg regwrite, output reg [1:0] Aluop ); always @(*) begin if (opcode == 7'b0000011) begin aluSrc = 1'b1; memtoreg = 1'b1; regwrite = 1'b1; memread = 1'b1; memwrite = 1'b0; branch = 1'b0; Aluop =2'b00; end else if (opcode == 7'b0100011) begin aluSrc = 1'b1; memtoreg = 1'bx; regwrite = 1'b0; memread = 1'b0; memwrite = 1'b1; branch = 1'b0; Aluop =2'b00; end else if (opcode == 7'b0110011) begin aluSrc = 1'b0; memtoreg = 1'b0; regwrite = 1'b1; memread = 1'b0; memwrite = 1'b0; branch = 1'b0; Aluop =2'b10; end else if (opcode == 7'b1100011) begin aluSrc = 1'b0; memtoreg = 1'bx; regwrite = 1'b0; memread = 1'b0; memwrite = 1'b0; branch = 1'b1; Aluop =2'b01; end else if (opcode == 7'b0010011) begin aluSrc = 1'b1; memtoreg = 1'b0; regwrite = 1'b1; memread = 1'b0; memwrite = 1'b0; branch = 1'b0; Aluop =2'b00; end else begin aluSrc = 1'b0; memtoreg = 1'b0; regwrite = 1'b0; memread = 1'b0; memwrite = 1'b0; branch = 1'b0; Aluop =2'b00; end if (stall == 1'b1) begin aluSrc = 1'b0; memtoreg = 1'b0; regwrite = 1'b0; memread = 1'b0; memwrite = 1'b0; branch = 1'b0; Aluop =2'b00; end end endmodule

pc.sv

/* verilator lint_off EOFNEWLINE */ /* verilator lint_off VARHIDDEN */ module pc( input logic [12:0] nxt_pc, output logic [12:0] pc_o, input logic clk, input stalling_signal ); integer counter; initial begin counter = 0; end always @(posedge clk) begin if(stalling_signal) begin pc_o <= pc_o; counter <= counter + 1; if(counter == 1) begin pc_o <= nxt_pc; counter <= 0; end end else begin pc_o <= nxt_pc; end end endmodule

IF_ID.sv

/* Name: Pipeline register between Datapath Fetch and Decode Stage */ module IF_ID (input logic clk, reset, clear, enable, input logic [31:0] InstrF, PCF, PCPlus4F, output logic [31:0] InstrD, PCD, PCPlus4D); always_ff @( posedge clk, posedge reset ) begin if (reset) begin // Asynchronous Clear InstrD <= 0; PCD <= 0; PCPlus4D <= 0; end else if (enable) begin if (clear) begin // Synchrnous Clear InstrD <= 0; PCD <= 0; PCPlus4D <= 0; end else begin InstrD <= InstrF; PCD <= PCF; PCPlus4D <= PCPlus4F; end end end endmodule

ID_IEx.sv

/* Name: Pipeline register between Datapath Decode and Execution Stage */ module ID_IEx (input logic clk, reset, clear, input logic [31:0] RD1D, RD2D, PCD, input logic [4:0] Rs1D, Rs2D, RdD, input logic [31:0] ImmExtD, PCPlus4D, output logic [31:0] RD1E, RD2E, PCE, output logic [4:0] Rs1E, Rs2E, RdE, output logic [31:0] ImmExtE, PCPlus4E); always_ff @( posedge clk, posedge reset ) begin if (reset) begin RD1E <= 0; RD2E <= 0; PCE <= 0; Rs1E <= 0; Rs2E <= 0; RdE <= 0; ImmExtE <= 0; PCPlus4E <= 0; end else if (clear) begin RD1E <= 0; RD2E <= 0; PCE <= 0; Rs1E <= 0; Rs2E <= 0; RdE <= 0; ImmExtE <= 0; PCPlus4E <= 0; end else begin RD1E <= RD1D; RD2E <= RD2D; PCE <= PCD; Rs1E <= Rs1D; Rs2E <= Rs2D; RdE <= RdD; ImmExtE <= ImmExtD; PCPlus4E <= PCPlus4D; end end endmodule

riscv_pip_27.sv

/* Name: RISC V RV32I Descrption: Implements 27 instructions 5 stage pipeline Hazard Unit is present for handling both control and data hazards */ module riscv_pip_27(input logic clk, reset, output logic [31:0] PCF, input logic [31:0] InstrF, output logic MemWriteM, output logic [31:0] ALUResultM, WriteDataM, input logic [31:0] ReadDataM); logic ALUSrcAE, RegWriteM, RegWriteW, ZeroE, SignE, PCJalSrcE, PCSrcE; logic [1:0] ALUSrcBE; logic StallD, StallF, FlushD, FlushE, ResultSrcE0; logic [1:0] ResultSrcW; logic [2:0] ImmSrcD; logic [3:0] ALUControlE; logic [31:0] InstrD; logic [4:0] Rs1D, Rs2D, Rs1E, Rs2E; logic [4:0] RdE, RdM, RdW; logic [1:0] ForwardAE, ForwardBE; controller c(clk, reset, InstrD[6:0], InstrD[14:12], InstrD[30], ZeroE, SignE, FlushE, ResultSrcE0, ResultSrcW, MemWriteM, PCJalSrcE, PCSrcE, ALUSrcAE, ALUSrcBE, RegWriteM, RegWriteW, ImmSrcD, ALUControlE); hazardunit h (Rs1D, Rs2D, Rs1E, Rs2E, RdE, RdM, RdW, RegWriteM, RegWriteW, ResultSrcE0, PCSrcE, ForwardAE, ForwardBE, StallD, StallF, FlushD, FlushE); datapath dp(clk, reset, ResultSrcW, PCJalSrcE, PCSrcE,ALUSrcAE, ALUSrcBE, RegWriteW, ImmSrcD, ALUControlE, ZeroE, SignE, PCF, InstrF, InstrD, ALUResultM, WriteDataM, ReadDataM, ForwardAE, ForwardBE, Rs1D, Rs2D, Rs1E, Rs2E, RdE, RdM, RdW, StallD, StallF, FlushD, FlushE); endmodule

hazardunit.sv

/* Name: Hazard Unit Description: Handles both Data Hazards and Control Hazards */ module hazardunit(input logic [4:0] Rs1D, Rs2D, Rs1E, Rs2E, input logic [4:0] RdE, RdM, RdW, input logic RegWriteM, RegWriteW, input logic ResultSrcE0, PCSrcE, output logic [1:0] ForwardAE, ForwardBE, output logic StallD, StallF, FlushD, FlushE); // RAW // Whenever source register (Rs1E, Rs2E) in execution stage matchces with the destination register (RdM, RdW) // of a previous instruction's Memory or WriteBack stage forward the ALUResultM or ResultW // And also only when RegWrite is asserted logic lwStall; always_comb begin ForwardAE = 2'b00; ForwardBE = 2'b00; if ((Rs1E == RdM) & (RegWriteM) & (Rs1E != 0)) // higher priority - most recent ForwardAE = 2'b10; // for forwarding ALU Result in Memory Stage else if ((Rs1E == RdW) & (RegWriteW) & (Rs1E != 0)) ForwardAE = 2'b01; // for forwarding WriteBack Stage Result if ((Rs2E == RdM) & (RegWriteM) & (Rs2E != 0)) ForwardBE = 2'b10; // for forwarding ALU Result in Memory Stage else if ((Rs2E == RdW) & (RegWriteW) & (Rs2E != 0)) ForwardBE = 2'b01; // for forwarding WriteBack Stage Result end // For Load Word Dependency result does not appear until end of Data Memory Access Stage // if Destination register in EXE stage is equal to souce register in decode stage // stall previous instructions until the the load word is avialbe at the writeback stage // Introduce One cycle latency for subsequent instructions after load word // There is two cycle difference between Memory Access and the immediate next instruction assign lwStall = (ResultSrcE0 == 1) & ((RdE == Rs1D) ¦ (RdE == Rs2D)); // assign FlushE = lwStall; assign StallF = lwStall; assign StallD = lwStall; // control hazard // whenever branch has been taken, we flush the following two instructions from decode and execute pip reg assign FlushE = lwStall ¦ PCSrcE; assign FlushD = PCSrcE; endmodule

datapath.sv

/* Name: 5 stage pipelined Datapath Description: Datapath with 5 stages Refer the microarchitecture in the images folder for reference */ module datapath(input logic clk, reset, input logic [1:0] ResultSrcW, input logic PCJalSrcE, PCSrcE, ALUSrcAE, input logic [1:0] ALUSrcBE, input logic RegWriteW, input logic [2:0] ImmSrcD, input logic [3:0] ALUControlE, output logic ZeroE, output logic SignE, output logic [31:0] PCF, input logic [31:0] InstrF, output logic [31:0] InstrD, output logic [31:0] ALUResultM, WriteDataM, input logic [31:0] ReadDataM, input logic [1:0] ForwardAE, ForwardBE, output logic [4:0] Rs1D, Rs2D, Rs1E, Rs2E, output logic [4:0] RdE, RdM, RdW, input logic StallD, StallF, FlushD, FlushE); logic [31:0] PCD, PCE, ALUResultE, ALUResultW, ReadDataW; logic [31:0] PCNextF, PCPlus4F, PCPlus4D, PCPlus4E, PCPlus4M, PCPlus4W, PCTargetE, BranJumpTargetE; logic [31:0] WriteDataE; logic [31:0] ImmExtD, ImmExtE; logic [31:0] SrcAEfor, SrcAE, SrcBE, RD1D, RD2D, RD1E, RD2E; logic [31:0] ResultW; logic [4:0] RdD; // destination register address // Fetch Stage mux2 jal_r(PCTargetE, ALUResultE, PCJalSrcE, BranJumpTargetE); mux2 pcmux(PCPlus4F, BranJumpTargetE, PCSrcE, PCNextF); flopenr IF(clk, reset, ~StallF, PCNextF, PCF); adder pcadd4(PCF, 32'd4, PCPlus4F); // Instruction Fetch - Decode Pipeline Register IF_ID pipreg0 (clk, reset, FlushD, ~StallD, InstrF, PCF, PCPlus4F, InstrD, PCD, PCPlus4D); assign Rs1D = InstrD[19:15]; assign Rs2D = InstrD[24:20]; regfile rf (clk, RegWriteW, Rs1D, Rs2D, RdW, ResultW, RD1D, RD2D); assign RdD = InstrD[11:7]; extend ext(InstrD[31:7], ImmSrcD, ImmExtD); // Decode - Execute Pipeline Register ID_IEx pipreg1 (clk, reset, FlushE, RD1D, RD2D, PCD, Rs1D, Rs2D, RdD, ImmExtD, PCPlus4D, RD1E, RD2E, PCE, Rs1E, Rs2E, RdE, ImmExtE, PCPlus4E); mux3 forwardMuxA (RD1E, ResultW, ALUResultM, ForwardAE, SrcAEfor); mux2 srcamux(SrcAEfor, 32'b0, ALUSrcAE, SrcAE); // for lui mux3 forwardMuxB (RD2E, ResultW, ALUResultM, ForwardBE, WriteDataE); mux3 srcbmux(WriteDataE, ImmExtE, PCTargetE, ALUSrcBE, SrcBE); adder pcaddbranch(PCE, ImmExtE, PCTargetE); // Next PC for jump and branch instructions alu alu(SrcAE, SrcBE, ALUControlE, ALUResultE, ZeroE, SignE); // Execute - Memory Access Pipeline Register IEx_IMem pipreg2 (clk, reset, ALUResultE, WriteDataE, RdE, PCPlus4E, ALUResultM, WriteDataM, RdM, PCPlus4M); // Memory - Register Write Back Stage IMem_IW pipreg3 (clk, reset, ALUResultM, ReadDataM, RdM, PCPlus4M, ALUResultW, ReadDataW, RdW, PCPlus4W); mux3 resultmux( ALUResultW, ReadDataW, PCPlus4W, ResultSrcW, ResultW); endmodule